SECTION 10 17 16

TELEPHONE ENCLOSURES

PART 1 GENERAL

1.01   SUMMARY

- Equipment and materials used shall be standard components that are manufactured and available for purchase as standard replacement parts as long as the product is commercially available from the manufacturer.

1.02   QUALITY ASSURANCE

- All tower installation, configurations, setup, program and related work shall be performed by electronic technicians thoroughly trained by the manufacturer in the installation and service of the equipment provided.

- The tower arm shall be warrantied against any defects in material and workmanship under normal use for a period of twenty-four (24) months from date of installation, provided that manufacturer receives a completed "Installation Certification" certifying the date on which the system has been installed.  An "Installation Certification" card shall be enclosed with every unit.  In the event that no "Installation Certification" is received by manufacturer, the twenty-four (24) months will commence on the date of shipment by the manufacturer.

PART 2 PRODUCTS

2.01   GENERAL

- The tower arm shall:

  - Consist of a light-weight aluminum tower arm with an integrated camera mount for attachment to an ETP-MTE free-standing aluminum emergency phone tower.

  - Have an integrated blue light mounting bracket that accommodates mounting the blue light directly above the camera.

2.02   HARDWARE

- The tower arm shall:

  - Be constructed of 0.125” thick aluminum and weigh approximately 9 lbs.

  - Measure:

    - Arm only: 10” W x 14.17” D x 43.42” H, with a 1” radius on each corner.

    - Arm on Tower, with Blue Light, and Aerators: 11.29” W x 14.17” D x 153.62” H.

  - Utilize a high-gloss, corrosion-inhibitive coating that shall be applied to withstand prolonged exposure to hard environments.

    - Tower arm shall be submerged in a Mil-Spec C-5541 Clear RoHS compliant anodyne bath to a thickness of 2-3 mils.

    - Tower arm shall be hand sanded for smoothness, welds inspected, and sharp edges removed if necessary.

    - Tower arm shall have a 2-4 mil layer of customer specified powder color applied.

    - Tower arm shall be cured at 400°F for 15 minutes before final visual inspection.

- The tower arm wiring access opening shall:

  - Measure 4.5”H x 2.5” W.

  - Have a flush cover plate that shall be held in place by 2 10-24 countersunk, tamper-resistant spanner screws.

- The tower arm shall be removable and held in place with eight 10-24 countersunk, tamper-resistant spanner screws.

- The tower arm shall have an integrated blue light mounting bracket for secure mounting of the blue light.

- The tower arm shall have a 1.5” NPT opening for attachment of a camera.

2.05   MANUFACTURED UNITS

- The tower arm shall be a Talkaphone ETP-MTE-ARM Emergency Telephone Tower Arm.

PART 3 EXECUTION

3.01      INSTALLATION

- The Contractor shall carefully follow instructions in documentation provided by the manufacturer to ensure all steps have been taken to provide a reliable, easy-to-operate system.

- All equipment shall be tested and configured in accordance with instructions provided by the manufacturer prior to installation.

END OF SECTION