SECTION 27 51 29

EMERGENCY COMMUNICATIONS SYSTEMS

PART 1 GENERAL

1.01   SUMMARY

- Equipment and materials used shall be standard components that are manufactured and available for purchase as standard replacement parts as long as the product is commercially available from the manufacturer.

1.02   QUALITY ASSURANCE

- All trim ring installation, configuration, setup, program and related work shall be performed by electronic technicians thoroughly trained by the manufacturer in the installation and service of the equipment provided.

- The trim ring shall be warrantied against any defects in material and workmanship under normal use for a period of sixty (60) months from date of installation, provided that manufacturer receives a completed "Installation Certification" certifying the date on which the system has been installed.  An "Installation Certification" card shall be enclosed with every unit.  In the event that no "Installation Certification" is received by manufacturer, the sixty (60) months will commence on the date of shipment by the manufacturer.

1.03   CERTIFICATIONS AND STANDARDS

- The trim ring as an assembly shall be designed to meet the following standards:

  - Safety

    - UL 60950-1

PART 2 PRODUCTS

2.01   GENERAL

- The trim ring shall:

  - Consist of an accessory for use when flush mounting an area of refuge (or area of rescue assistance) command unit.

2.02   HARDWARE

- The trim ring shall:

  - Be constructed of 16 Ga. cold-rolled steel (CRS).

  - Be painted black with a flat texture finish.

  - Be constructed of two (2) individual pieces.

  - Measure 16.31” W x 45.38” H x 0.06” D when fully assembled.

  - Have four 0.150” mounting holes on each individual piece of the trim ring—for a total of eight (8) mounting holes when fully assembled.

  - The trim ring shall be hand sanded for smoothness, welds inspected, and sharp edges removed, if necessary.

- The trim ring shall have an opening in the front to accommodate flush mounting an area of refuge (or area of rescue assistance) command unit.  The area of refuge (or area of rescue assistance) command unit opening shall:

  - Measure 43.38” H x 14.31” W when fully assembled.

2.04   MANUFACTURED UNITS

- The trim ring shall be a Talkaphone AOR-TR32 Trim Ring for Flush Mounting Accessory.

PART 3 EXECUTION

3.01      INSTALLATION

- The Contractor shall carefully follow instructions in documentation provided by the manufacturer to ensure all steps have been taken to provide a reliable, easy-to-operate system.

- All equipment shall be tested and configured in accordance with instructions provided by the manufacturer prior to installation.

END OF SECTION