SECTION 10 17 16

TELEPHONE ENCLOSURES

PART 1 GENERAL

1.01   SUMMARY

- Equipment and materials used shall be standard components that are manufactured and available for purchase as standard replacement parts as long as the product is commercially available from the manufacturer.

1.02   QUALITY ASSURANCE

- All tower installation, configurations, setup, program and related work shall be performed by electronic technicians thoroughly trained by the manufacturer in the installation and service of the equipment provided.

- The tower shall be warrantied against any defects in material and workmanship under normal use for a period of five (5) years from date of installation, provided that manufacturer receives a completed "Installation Certification" certifying the date on which the system has been installed.  An "Installation Certification" card shall be enclosed with every unit.  In the event that no "Installation Certification" is received by manufacturer, the five (5) years will commence on the date of shipment by the manufacturer.

- The blue light and faceplate light shall be warrantied against any defects in material and workmanship under normal use for a period of twenty-four (24) months from date of installation, provided that manufacturer receives a completed "Installation Certification" certifying the date on which the system has been installed.  An "Installation Certification" card shall be enclosed with every unit.  In the event that no "Installation Certification" is received by manufacturer, the twenty four (24) months will commence on the date of shipment by the manufacturer.

1.03   CERTIFICATIONS AND STANDARDS

- The tower as an assembly shall be certified to:

  - UL Std 60950-1

- The included LED blue light (model: ETP-EL or ETP-EL12/24) shall be certified to:

  - UL Std 1598

  - CSA Std C22.2 No. 250.0

PART 2 PRODUCTS

2.01   GENERAL

- The tower shall:

  - Consist of a highly vandal-resistant free-standing steel emergency phone tower mount with an integrated flashing LED blue light.

  - Have an integrated LED faceplate light, mounted directly above the phone faceplate.

  - Have an integrated vandal resistant polycarbonate lens cover and camera mounting bracket.

2.02   HARDWARE

- The tower shall:

  - Be constructed of 0.25” thick steel and weigh approximately 340 lbs.

  - Measure:

    - Tower only: 12” W x 10” D x 108” H, with a 2” radius on each corner.

  - Utilize a high-gloss, multi-coat, corrosion-inhibitive coating that shall be applied to withstand prolonged exposure to hard environments.

    - Tower shall be sandblasted to SSPC-6 standards before a 2-3 mil layer of rust-inhibitive primer is applied.

    - Tower shall be hand sanded for smoothness before a second 2-3 mil layer of primer is applied.

    - Tower shall have a 2-3 mil layer of customer specified color coat applied.

    - Tower shall have a 1-2 mil layer of clear coat applied.

- The tower base plate shall:

  - Be 2.0” above the tower base.

  - Be constructed of 0.75” A-36 Structural Steel.

  - Have a 4” diameter center hole for wiring access.

  - Have four 1” x 2.31” slots for anchor bolt attachment.

- The tower wiring access opening shall:

  - Measure 9”H x 6.75” W.

  - Be located 15” above the base of the tower.

  - Have a flush cover plate that shall:

    - Be constructed of 0.25” thick steel.

    - Be held in place by two 1/4" countersunk, tamper-resistant spanner screws.

- The tower shall have an opening in the front to accommodate flush mounting an emergency phone. The phone opening shall:

  - Measure 10” H x 6.75” W.

  - Have six self-clinching 10-24 stainless steel threaded nuts to mount the emergency phone.

- Directly below the tower phone opening, the tower shall have a section with a 30° downward slope from rear to front, spanning the depth of the phone opening to the full tower depth.

- The tower shall have the word “EMERGENCY” emblazoned on all four sides in 3.25” high reflective white letters. Custom lettering, sizes and colors are available.

- The tower blue light shall:

  - Be mounted within an enclosed protective polycarbonate housing.

  - Have a polycarbonate refractor lens assembly with a prismatic pattern to increase visibility at greater distances.

- The tower camera mount shall:

  - Have a vandal resistant 2.81” diameter polycarbonate lens cover.

  - Have an adjustable bracket with a slot for a 1/4" – 20 UNC camera mount.

2.03   FUNCTIONALITY

- Blue Light

  - The blue light shall remain lit at all times.

  - The blue light shall automatically flash 78 times per minute when triggered by the emergency phone.

  - The blue light shall have an illumination rating of 209 lumens (peak).

  - The blue light shall retain 70% of its initial output intensity after 50,000 hours of operation.

- Faceplate Light

  - The faceplate light shall remain lit at all times.

  - The faceplate light shall have a concealed, ultra-bright LED design.

  - The faceplate light LEDs shall have no less than 50,000 hour lifetime.

2.04   POWER REQUIREMENTS

- The tower shall be powered by one of the following power sources:

  - 12VDC – Nominal: 9 Watts

  - 24VDC – Nominal: 9 Watts

  - 24VAC – Nominal: 9 Watts

  - 120VAC – Nominal: 9 Watts

2.05   MANUFACTURED UNITS

- The tower shall be a Talkaphone ETP-MT/R-OP2 Emergency Telephone Tower.

PART 3 EXECUTION

3.01      INSTALLATION

- The Contractor shall carefully follow instructions in documentation provided by the manufacturer to ensure all steps have been taken to provide a reliable, easy-to-operate system.

- All equipment shall be tested and configured in accordance with instructions provided by the manufacturer prior to installation.

- The tower shall include 24-inch long J-bolts for mounting into a 24” x 24” concrete foundation. Depth to vary according to local regulations and other site-specific considerations.

- The J-bolts shall protrude approximately 5 inches from the surface of the foundation.

END OF SECTION